WS-501R  is a good washing agent for cotton or its blends dyed with reactive dyestuff.

And it shows good efficiency on improvement of wet fastness and a migration prevention of unreacted/hydrolysed dyestuffs.

| ◈ SPECIFICATION - APPEARANCE - IONIC NATURE - pH (1% aq. Sol'n) - SP.GR(at 25℃) - SOLUBILITY - STORAGE STABILITY - COMPATABILITY ◈ FEATURES | Light Yellowish Liquid Nonionic 7.0 ± 1.0 1.05 ± 0.05 Easily soluble in cold water. Min. 1 year under cool and dry conditions when stored in original sealed, packaging. Compatible with anionic and nonionic materials. - WS-501R can reduce the number of times of a washing process more than once because it has excellent washing effect. Therefore it can show effect of both shortening of work time and energy economy. - Because WS-501R has more excellent washing effect than surfactant, it removes residue of unreacted/hydrolysed dyestuffs to the maximum in a short time. Therefore it shows good effect on increasing wet fastness and in a washing process of printed product. - WS-501R doesn't cause reversible reaction and hydrolysis toward to fixed dyestuffs. And it does not adversely affect various kinds of fastnesses. - Because its aq.sol'n forms uniform colloidal solution, WS-501R acts as a solvent of fallen dyestuff particles. It hasn't affinity to textile. Therefore it shows excellent effect on preventing redeposition or on migrating unfixed dyestuffs residue on fiber. |
| --- | --- |

| ◈ APPLICATION FIBERS ◈ HOW TO USE ◈ RECOMMENDED RECIPE | - WS-501R is a good washing agent and a preventing agent of migration of unfixed dyestuffs residue on fiber in a printing process. - Because of its low foaming property, WS-501R doesn't cause trouble by foam. Especially it is effective on a continuous washing process and on Winch, Jigger and Cheese Processes too. - It is environmental-friendly product as APEO-free and NPEO-free. All fibers such as Polyester, Nylon, Cotton, etc. Recommended use levels are 1-4 g/ℓ according to kinds of dyeing materials, the depth of color and the bath ratio, etc. ※ Recommended recipe for washing of dyeing materials 1) High bath ratio |  |
| --- | --- | --- |
|  | - 98% NaOH - WS-501R - Washing conditions - Washing hot conditions - Washing cold conditions | : 0.5-1 g/ℓ : 1-2 g/ℓ : Washing for 30 min at 85-100℃ : Washing hot for 30 min at 85-100℃ : Washing cold for 15-30 min at room temperature. |
|  | 2) Low bath ratio |  |
|  | - 98% NaOH - WS-501R - Washing conditions - Washing hot conditions - Washing cold conditions | : 0.5-1 g/ℓ : 2-3 g/ℓ : Washing for 30 min at 85-100℃ : Washing hot for 30 min at 85-100℃ : Washing cold for 15-30 min at room temperature. |

| ◈ RECOMMENDED RECIPE | 3) In case of dark color |  |
| --- | --- | --- |
|  | - 98% NaOH - WS-501R - Washing conditions - Washing hot conditions - Washing cold conditions | : 0.5-1 g/ℓ : 2-4 g/ℓ : Washing for 30 min at 85-100℃ : Washing hot for 30 min at 85-100℃ : Washing cold for 15-30 min at room temperature. |
|  | 4) In case of medium color |  |
|  | - 98% NaOH - WS-501R - Washing conditions - Washing hot conditions - Washing cold conditions | : 0.5-1 g/ℓ : 1-2 g/ℓ : Washing for 30 min at 85-100℃ : Washing hot for 30 min at 85-100℃ : Washing cold for 15-30 min at room temperature. |
| ※ The method to improve washing fastness, perspiration fastness and sea water fastness of cellulose-based fibers such as cotton, etc. - Treat with MONOREX-NRD after washing with WS-501R . - Please refer to its catalog to treat with MONOREX-NRD. ※ Recommended recipe for washing of printing materials |  |  |
|  | - 98% NaOH - WS-501R - Washing conditions - Washing hot conditions - Washing cold conditions | : 0.5-1 g/ℓ : 2 g/ℓ : Washing for 60 min at 95℃ : Washing hot for 30 min at 85-100℃ : Washing cold for 15-30 min at room temperature. |
| ◈ PACKAGE | 120 Kgs, 200 Kgs NET IN DRUMS. |  |